Hydrology Protocols

Below is a list of the protocols to follow in doing an ARCGIS hydrology analysis. These protocols include the following: usage of flow direction, accumulation, and stream generation. To follow this protocol, a DEM (digital elevation model) is necessary.

- Step #1: Computing the Sinks

  - Sinks are areas of a DEM that did not generate properly or have inconclusive data. This means that to generate a proper model of flow direction and accumulation, the sinks need to be filled first.

    - Open the Flow Direction Tool, select the DEM as the input, and run the tool and save as Flow.

    - Open the Sink Tool, select the Flow as the input, and run the tool and save as Sinks

    - To fill the sinks, open the Fill Tool and select the original DEM as the input, run the tool and save it as Sinks_Filled

- Step #2: Computing the Flow Direction

  - From here we can calculate the proper flow direction based on the DEM that has had the sinks filled in, for a more accurate model.

    - Open the Flow Direction Tool again, select Sinks_Filled as the input, run the tool and save as Flow_Sinks_Filled

- Step #3: Computing Flow Accumulation

  - The next step is creating a visualization of the flow accumulation, or the rate of flow around the digital elevation model.

    - Open the Flow Accumulation Tool and select the Flow_Sinks_Filled as the input, run the tool and save as Flow_Accumulation

      - Click on the Flow_Accumulation layer and open the symbology menu. Under “Stretch Type” change this to Standard Deviation and save.

- Step #4: Extracting Raster Streams

  - Extracting Raster Streams is the first step towards visualizing the various flows of water around the site by grouping them into streams and data that can be edited.

    - Open the Raster Calculator under Spatial Analysis Tools. Type the following expression “Flow_Accumulation” >= 5000. Save the output as Streams

    - Open the Stream Link Tool and set Streams as the input stream raster and use Flow_Sinks_Filled. Run the tool and save as Streams_ID

    - Each stream can be given a unique color/identifier my clocking on the Raster Layer and opening Symbology, and setting it to “Unique Values”

- Step #4: Extracting Vector Streams

  - Vector visualization is an easier way to visualize water movement and is a more realistic representation of streams

    - Open the Stream to Feature Tool and select Streams_ID as the input Stream Raster. Next select the Flow_Sinks_Filled. Uncheck “Simplify Polylines.” And save as Streams_Vector.

- Step #5: Extracting Watersheds

  - The final step is finding the subbasins of the streams which can be used for further hydrological analysis.

    - Open the Watershed Tool and select Flow_Sinks_Filled as the input Flow Direction Raster and select Streams_ID as the input Raster. Run the tool and save as Subbasins

With these layers completed, the door to other hydrological analysis is open, but requires a large amount of experience and fidelity.